3. Features

- Compatible with DFRobot FireBeetle V2 Series

- Small Size of 25.4×60 mm

- ESP32 Dual-core low power maincontroller, WiFi+BT4.0

- GDI Display Port, esay to connect

- Onboard Charging Circuit and PH2.0 li-ion Battery Port

4. Specification

- Operating Voltage: 3.3V

- Input Voltage: 3.3V-5.5V (Support USB Charging)

- Processor: Tensilica LX6 dual-core processor (One for high-speed connection; one for independent application development)

- Main Frequency: 240MHz

- SRAM: 520KB

- Flash: 32Mbit

- Wi-Fi Standard: FCC/CE/TELEC/KCC

- Wi-Fi Protocol: 802.11 b/g/n/d/e/i/k/r (802.11n，speed up to 150 Mbps), A-MPDU and A-MSDU Aggregation, support 0.4us guard interval)

- Frequency Range: 2.4-2.5 GHz

- Bluetooth Protocol: Bluetooth v4.2 BR/EDR and BLE standard compliant

- Bluetooth Audio: CVSD and SBC audio

- On-chip Clock: 40MHz crystal, 32.768KHz crystal

- Digital I/O x18

- Analog Input x11

- SPI x1

- IIC x1

- I2S x1

- RGB_LED: WS2812

- Connector: FireBeetle V2 series compatible

- Operating Temperature: -20℃ to +85℃

- Module Size: 25.4 × 60(mm)

- Mount Hole Size: M2, diameter 2.0mm

- Weight: 13g

THW_1060_Regler:

THW_1060_Regler:

Es wurde F/R Mode gewählt. Das heisst --> Vorwärts und Rückwärts ( Ohne Bremsung )

BATT: LIPO Baterie.

Motor Auswahl:

Für unsere Aufbau ist sinnvoll:

. 2S Lipo (7.4 V, 30-50C, 2200-3000 mAh)

  - passt perfekt zum Motor (Nennspannung 6-8 V )

  - Genug Spannung, um den Motor im Effizienzbereich laufen zu lassen.

  - mit 2200 mAh bei 20 A Last 6-7 min Laufzeit.

RS_540/545H: